Baremetal Bird – Open-Source Quadcopter Platform (From-Scratch Design)

System Bill of Materials

Document Number: DRONE-000-BOM

Revision A

Date: 11/16/2025

Prepared by: Omar Sameh

| Category | Part Name | Part Number / Spec | Quantity | Unit Cost (EGP) | Total (EGP) | Link |
| --- | --- | --- | --- | --- | --- | --- |
| Mechanical | FDM-Printed F450 Frame | Villamany F450 | 1 set | 1425 | 1425 | https://www.thingiverse.com/thing:1206960 |
| Mechanical | Propeller Guards 1045 | – | 4 | Included | – | https://www.thingiverse.com/thing:6434234 |
| Mechanical | M3×10 Screws | – | 16 | 0 | 0 |  |
| Mechanical | M3×20 Screws | – | 8 | 0 | 0 |  |
| Mechanical | M3 Brass Insert | M3×4×5 | 28 | 4 | 112 | https://ampere-electronics.com/product/m3-knurled-brass-threaded-insert-nut/ |
| Power | DXF Battery 3S 5200mAh 80C | DXF 3S-5200-80C | 2 | 2000 | 4000 | https://dxf-hobby.store/products/frdxf-lipo-battery-3s-11-1v-5200mah-80c-blue-series-lipo-hardcase-w-3 |
| Power | BBX1 LiPo Tester | BBX1-8S | 1 | 125 | 125 | https://makerselectronics.com/product/bbx1-8s-li-po-li-ion-voltage-tester/ |
| Power | T-Type Connector | – | 4 | 25 | 100 | https://makerselectronics.com/product/t-type-battery-connector-male-female/ |
| Power | Mini-360 Buck | Mini-360 | 1 | 30 | 30 | https://makerselectronics.com/product/dc-dc-step-down-converter-3a-ultra-mini-360-4-75v23vdc-to-1v17vdc/ |
| Power | MP1584EN Buck | MP1584EN | 1 | 45 | 45 | https://makerselectronics.com/product/mp1584en-mini-dc-dc-buck-step-down-module-4-5v-28v-input/ |
| Power | NTC Thermistor 10k | IP65 10kΩ | 2 | 5 | 10 | https://makerselectronics.com/product/ntc-thermistor-waterproof-ip65-10k%cf%89/ |
| Power | Imax B6 Mini Charger | SkyRC B6 Mini | 1 | 3000 | 3000 | https://www.ram-e-shop.com/shop/imax-b6-mini-skyrc-imax-b6mini-professional-balance-charger-discharger-7673 |
| Power | 10AWG Cu Wire | For the battery Power distribution | 10cm | _ | _ |  |
| Power | 14AWG Cu Wire | For ESCs power distribution | 40cm | _ | _ |  |
| Propulsion | Brushless Motor | A2212-1400KV | 4 | 340 | 1360 | https://ampere-electronics.com/product/a2212-brushless-motor-1400kv/ |
| Propulsion | Simonk ESC 30A | Simonk-30A | 4 | 435 | 1740 | https://makerselectronics.com/product/simonk-esc-speed-controller-for-brushless-motor-30a-with-t-type-connector-motor-connectors/ |
| Propulsion | 8045 Propeller Pair | 8045 GF | 2 pairs | 200 | 400 | https://microohm-eg.com/product/1pair-8045-8x4-5-inch-cw-propeller-ccw-prop-for-rc-multicopter-f450-quadcopter-black/ |
| Flight Controller | ATmega328P-PU | ATmega328P-PU | 1 | 285 | 285 | https://makerselectronics.com/product/atmega328p-pu-microchip-avr-microcontroller/ |
| Flight Controller | ZIF Socket | 28-pin | 1 | 30 | 30 | https://makerselectronics.com/product/universal-zif-socket-28-pins/ |
| Flight Controller | IC Socket | 28-pin | 1 | 3 | 3 | https://makerselectronics.com/product/ic-socket-1414-base-28-pin/ |
| Flight Controller | PL2303 USB-TTL | PL2303 | 1 | 60 | 60 | https://makerselectronics.com/product/pl2303-usb-ttl-converter-module/ |
| Flight Controller | Female Headers | 40-pin | 2 | 5 | 10 | https://makerselectronics.com/product/pin-headers-female-2-54mm-40-pin/ |
| Flight Controller | BNO055 IMU | BNO055 | 1 | 3100 | 3100 | https://devboardsmarket.com/products/adafruit-9-dof-absolute-orientation-imu-fusion-breakout-bno055 |
| Flight Controller | MS5611 Barometer | MS5611 | 1 | 450 | 450 | https://makerselectronics.com/product/gy-63-ms5611-01ba03-barometer-pressure-sensor-module/ |
| Flight Controller | ISP Connector | 10-pin | 2 | 20 | 40 | https://makerselectronics.com/product/avr-isp-10pin-to-icsp-6pin-converter2/ |
| Flight Controller | 20 MHz Crystal | HC-49S | 2 | 6 | 6 | https://makerselectronics.com/product/crystal-20-000mhz-hc-49s-20ppm/ |
| Flight Controller | 470µF Capacitor | 35V | 4 | 12 | 12 | https://makerselectronics.com/product/capacitor-470uf-35v-1016mm/ |
| Flight Controller | 22pF Capacitor | 50V | 8 | 4 | 4 | https://makerselectronics.com/product/ceramic-capacitor-22pf-50v/ |

Custom BMS
The battery-management system isn’t defined yet. Before it can be designed or priced, the battery configuration needs to be fixed, along with the balancing method, protection thresholds, and the MOSFET/shunt arrangement. None of these parameters have been decided, so the BMS is still open.

Power Distribution Board
The PDB layout is not specified. The copper weight, trace or busbar dimensions, connector type, and mounting pattern all need to be set. Without these, a board design or cost estimate can’t be produced. We will use the wires (10AWG and 14AWG) in the prototype phase.

Flight Controller PCB
Only the components are listed. The actual schematic, power-regulation layout, grounding strategy, and connector placement are not defined. Layer count, board size, and sensor routing are also missing. The PCB can’t proceed until those details are locked down.

Bearing and Rod Assembly (Thrust Stand)
The thrust stand requires defined rod diameter and material, bearing dimensions, and mounting geometry. None of these values have been chosen, so the assembly remains undefined.

Prop-Guard Printing
All printed mechanical parts, including the prop guards, are already accounted for in the 1425 EGP total. No additional cost or work is pending here.

RC Control Link
The radio link will use a standard 2.4 GHz quadcopter controller such as FlySky. The receiver will output PWM or iBUS/PPM signals for throttle, pitch, roll, and yaw. No custom communication protocol is required.